VGA Snake 游戏具体功能实现说明与代码分析

本文档根据当前工程源码生成，重点说明敌人的随机长度、随机位置和随机运动，食物按比例随机出现，玩家身体变长与移动，GAMEOVER 显示，画面颜色控制，VGA 信号输出，碰撞判定，以及若干附加功能实现。

工程目录：D:\vivadoproject\vga_snake

1. 工程总体结构

核心逻辑分为五层：板级封装 board_top.v、Vivado block design wrapper、游戏顶层 top.v、游戏状态机 snake_game.v、像素/颜色渲染 grid_renderer.v 与 VGA 时序 vga_timing.v。

| 文件 | 主要作用 |
| --- | --- |
| vga_snake.srcs/sources_1/new/snake_game.v | 维护蛇、食物、敌人、分数、生命、状态机和碰撞；向渲染模块提供 cell_state。 |
| vga_snake.srcs/sources_1/new/grid_renderer.v | 把当前像素坐标转换为网格坐标，并按 cell_state 输出 RGB 颜色。 |
| vga_snake.srcs/sources_1/new/vga_timing.v | 产生 640x480@60Hz 的 hsync、vsync、active_video、pixel_x、pixel_y。 |
| vga_snake.srcs/sources_1/new/top.v | 连接按键、tick、snake_game、grid_renderer、vga_timing、GAMEOVER 覆盖显示、数码管。 |
| vga_snake.srcs/sources_1/new/board_top.v | 板级入口，连接 vga_snake_wrapper 与 VGA/数码管/按键/时钟/复位等外设。 |
| vga_snake.srcs/constrs_1/new/nexys_ddr4_vga.xdc | 把 hsync、vsync、vga_r/g/b、sys_clock、reset 等逻辑端口绑定到 Nexys DDR4 管脚。 |

snake_game 模块参数与状态编码（vga_snake.srcs/sources_1/new/snake_game.v:10-49）

10: module snake_game #(

11:     parameter COLS = 40,

12:     parameter ROWS = 30,

13:     parameter MAXLEN = (COLS*ROWS),

14:     parameter MAX_ENEMIES = 3,

15:     parameter ENEMY_MIN_LEN = 4,

16:     parameter ENEMY_MAX_LEN = 8,

17:     parameter ENEMY_SPAWN_MIN_TICKS = 40,

18:     parameter ENEMY_SPAWN_RANDOM_MASK = 16'h003F,

19:     parameter FOOD_SPAWN_MIN_TICKS = 8,

20:     parameter FOOD_SPAWN_RANDOM_MASK = 16'h000F,

21:     parameter INITIAL_LIVES = 1,

22:     parameter MAX_LIVES = 9,

23:     parameter INVINCIBLE_TICKS = 20

24: )(

25:     input  wire        clk,

26:     input  wire        rst_n,

27:     input  wire [1:0]  dir,       // 00 up, 01 down, 10 left, 11 right

28:     input  wire        dir_valid,

29:     input  wire        start,

30:     input  wire        pause,

31:     input  wire        reset_game,

32:     input  wire        tick,

33:     input  wire [2:0]  enemy_limit,

34:     input  wire [5:0]  cell_x,

35:     input  wire [4:0]  cell_y,

36:     output wire [2:0]  cell_state,

37:     output reg  [15:0] score,

38:     output reg  [3:0]  lives,

39:     output reg  [1:0]  state      // 0 idle,1 running,2 pause,3 gameover

40: );

41:

42:     localparam EMPTY       = 3'b000;

43:     localparam BODY        = 3'b001;

44:     localparam HEAD        = 3'b010;

45:     localparam RED_FOOD    = 3'b011;

46:     localparam YELLOW_FOOD = 3'b100;

47:     localparam BLUE_FOOD   = 3'b101;

48:     localparam ENEMY_HEAD  = 3'b110;

49:     localparam ENEMY_BODY  = 3'b111;

cell_state 是整个显示链路的关键编码：snake_game 按每个格子返回 EMPTY、BODY、HEAD、不同食物、敌人头/身体；grid_renderer 再把这些状态翻译为 RGB。

2. 随机数来源：LFSR 伪随机序列

硬件里通常不用软件 rand()。本工程在 snake_game.v 中使用 16 位 LFSR。每个时钟周期用反馈多项式推进一次，后续的敌人长度、敌人位置、敌人方向、食物种类和生成延迟都从 lfsr 的不同位或取模结果获得。

LFSR 初始化与推进（vga_snake.srcs/sources_1/new/snake_game.v:642-645）

642:             hy <= 0;

643:             cur_dir <= DIR_RIGHT;

644:             next_dir <= DIR_RIGHT;

645:             lfsr <= 16'hACE1;

LFSR 每周期更新（vga_snake.srcs/sources_1/new/snake_game.v:711-712）

711:             end else begin

712:                 lfsr <= {lfsr[14:0], lfsr[15] ^ lfsr[13] ^ lfsr[12] ^ lfsr[10]};

分析：LFSR 是确定性伪随机序列。复位后种子固定为 16'hACE1，但 reset_game 时会与常数异或，游戏中又不断滚动，所以玩家体验上表现为随机。

3. 敌人的随机长度、随机位置和随机运动

3.1 随机长度

敌人长度由 ENEMY_MIN_LEN 和 ENEMY_MAX_LEN 限定，random_enemy_len(seed) 先计算长度跨度 span，再用 seed % span 落到范围内。当前 top.v 传入的范围是 4 到 8，因此敌人长度为 4、5、6、7、8 中的一个。

敌人长度与生成延迟函数（vga_snake.srcs/sources_1/new/snake_game.v:512-525）

512:     function [3:0] random_enemy_len;

513:         input [15:0] seed;

514:         integer span;

515:         begin

516:             span = ENEMY_MAX_LEN - ENEMY_MIN_LEN + 1;

517:             random_enemy_len = ENEMY_MIN_LEN + (seed % span);

518:         end

519:     endfunction

520:

521:     function [15:0] next_spawn_delay;

522:         input [15:0] seed;

523:         begin

524:             next_spawn_delay = ENEMY_SPAWN_MIN_TICKS + (seed & ENEMY_SPAWN_RANDOM_MASK);

525:         end

例子：若 ENEMY_MIN_LEN=4、ENEMY_MAX_LEN=8，span=5；seed%5 为 0 时长度为 4，为 4 时长度为 8。

3.2 随机位置

敌人生成不是直接把随机坐标写进去，而是先用 lfsr % GRID_CELLS 得到一个候选格 enemy_scan_idx，然后检查从这个格子向左摆放 qlen 个身体段是否全部合法。若不合法，enemy_scan_idx 逐格加 1 继续扫描，直到找到可用位置。

敌人生成位置合法性检查（vga_snake.srcs/sources_1/new/snake_game.v:379-387）

379:     function spawn_cell_free;

380:         input [15:0] qidx;

381:         begin

382:             spawn_cell_free =

383:                 (qidx < GRID_CELLS) &&

384:                 (qidx != head_idx) &&

385:                 !snake_map[qidx] &&

386:                 !food_occupies_index(qidx) &&

387:                 !enemy_occupies_index(qidx);

敌人按长度从候选格向左生成（vga_snake.srcs/sources_1/new/snake_game.v:976-1000）

976:                         end else if (pending_enemy_spawn) begin

977:                             spawn_slot = first_dead_enemy_slot(1'b0);

978:                             if (spawn_slot == NO_ENEMY_SLOT) begin

979:                                 pending_enemy_spawn <= 1'b0;

980:                                 enemy_spawn_countdown <= next_spawn_delay(lfsr);

981:                             end else begin

982:                                 spawn_len = random_enemy_len(lfsr ^ enemy_scan_idx);

983:                                 if (enemy_spawn_valid(enemy_scan_idx, spawn_len)) begin

984:                                     enemy_base = spawn_slot * ENEMY_MAX_LEN;

985:                                     enemy_alive[spawn_slot] <= 1'b1;

986:                                     enemy_dir[spawn_slot] <= DIR_RIGHT;

987:                                     enemy_len[spawn_slot] <= spawn_len;

988:                                     for (ej = 0; ej < ENEMY_MAX_LEN; ej = ej + 1) begin

989:                                         if (ej < spawn_len)

990:                                             enemy_pos[enemy_base + ej] <= enemy_scan_idx - ej;

991:                                         else

992:                                             enemy_pos[enemy_base + ej] <= INVALID_INDEX;

993:                                     end

994:                                     pending_enemy_spawn <= 1'b0;

995:                                     enemy_spawn_countdown <= next_spawn_delay(lfsr ^ enemy_scan_idx);

996:                                 end else if (enemy_scan_idx >= GRID_CELLS - 1) begin

997:                                     enemy_scan_idx <= 0;

998:                                 end else begin

999:                                     enemy_scan_idx <= enemy_scan_idx + 1'b1;

1000:                                 end

分析：spawn_cell_free 会排除玩家头、玩家身体、食物和已有敌人；enemy_spawn_valid 还要求 qx >= qlen - 1，避免向左摆身体时跨出边界。因此“随机位置”不是盲放，而是随机起点加合法扫描。

3.3 随机运动

每个敌人在移动 tick 中从 lfsr 的不同两位取 rand_dir。若随机方向正好与当前方向相反，则改为向右转，避免敌人直接 180 度回头。随后 enemy_move_valid 检查下一格是否在边界内、是否撞到敌人自己/其他敌人、是否撞到食物。

敌人移动合法性函数（vga_snake.srcs/sources_1/new/snake_game.v:342-351）

342:     function enemy_move_valid;

343:         input [2:0] slot;

344:         input [1:0] move_dir;

345:         reg [15:0] qidx;

346:         begin

347:             qidx = next_grid_index(enemy_pos[slot * ENEMY_MAX_LEN], move_dir);

348:             enemy_move_valid =

349:                 (qidx != INVALID_INDEX) &&

350:                 !enemy_occupies_index_except_self_tail(slot, qidx) &&

351:                 !food_occupies_index(qidx);

敌人随机方向、碰撞玩家与身体平移（vga_snake.srcs/sources_1/new/snake_game.v:1171-1216）

1171:                         if (!player_gameover && !player_damaged) begin

1172:                             for (ei = 0; ei < MAX_ENEMIES; ei = ei + 1) begin

1173:                                 enemy_base = ei * ENEMY_MAX_LEN;

1174:                                 if (enemy_alive[ei] &&

1175:                                     !(player_hit_enemy_body && (killed_enemy_slot == ei[2:0]))) begin

1176:                                     case (ei)

1177:                                         0: rand_dir = lfsr[1:0];

1178:                                         1: rand_dir = lfsr[3:2];

1179:                                         2: rand_dir = lfsr[5:4];

1180:                                         3: rand_dir = lfsr[7:6];

1181:                                         default: rand_dir = lfsr[9:8];

1182:                                     endcase

1183:

1184:                                     if (is_opposite_dir(enemy_dir[ei], rand_dir))

1185:                                         try_dir = turn_right(enemy_dir[ei]);

1186:                                     else

1187:                                         try_dir = rand_dir;

1188:

1189:                                     enemy_next_idx = next_grid_index(enemy_pos[enemy_base], try_dir);

1190:                                     enemy_can_move = enemy_move_valid(ei[2:0], try_dir);

1191:                                     if (enemy_can_move) begin

1192:                                         enemy_hit_player =

1193:                                             (enemy_next_idx == nidx) ||

1194:                                             (enemy_next_idx == head_idx) ||

1195:                                             (snake_map[enemy_next_idx] &&

1196:                                              (player_grew || (enemy_next_idx != tailidx)));

1197:

1198:                                         if (enemy_hit_player && (invincible_ticks == 0)) begin

1199:                                             if (lives <= 1) begin

1200:                                                 lives <= 0;

1201:                                                 state <= 3;

1202:                                                 player_gameover = 1'b1;

1203:                                             end else begin

1204:                                                 lives <= lives - 1'b1;

1205:                                                 invincible_ticks <= INVINCIBLE_TICKS;

1206:                                                 invincible_blink <= 1'b0;

1207:                                                 player_damaged = 1'b1;

1208:                                             end

1209:                                         end

1210:

1211:                                         for (ej = ENEMY_MAX_LEN - 1; ej > 0; ej = ej - 1) begin

1212:                                             enemy_pos[enemy_base + ej] <= enemy_pos[enemy_base + ej - 1];

1213:                                         end

1214:                                         enemy_pos[enemy_base] <= enemy_next_idx;

1215:                                         enemy_dir[ei] <= try_dir;

1216:                                     end

分析：enemy_pos[base] 是敌人头，enemy_pos[base+1..] 是身体。移动时从尾到头依次复制上一节位置，最后把 enemy_pos[base] 写成 enemy_next_idx，这和玩家蛇的“头前进、身体跟随”思路一致。

4. 食物按比例随机出现

食物有三种：红色食物、黄色食物、蓝色生命食物。weighted_food_type(seed) 用 seed%10 划分概率桶：0-4 为红色，5-7 为黄色，8-9 为绿色/蓝色生命食物。因此理论比例是 5:3:2。

食物类型按 5:3:2 权重选择（vga_snake.srcs/sources_1/new/snake_game.v:250-284）

250:     function [1:0] weighted_food_type;

251:         input [15:0] seed;

252:         integer bucket;

253:         begin

254:             bucket = seed % 10;

255:             if (bucket < 5)

256:                 weighted_food_type = FOOD_RED;

257:             else if (bucket < 8)

258:                 weighted_food_type = FOOD_YELLOW;

259:             else

260:                 weighted_food_type = FOOD_GREEN;

261:         end

262:     endfunction

263:

264:     function [1:0] select_food_type;

265:         input [15:0] seed;

266:         reg [1:0] chosen;

267:         begin

268:             chosen = weighted_food_type(seed);

269:             if (food_type_available(chosen)) begin

270:                 select_food_type = chosen;

271:             end else if (!food_valid) begin

272:                 select_food_type = FOOD_RED;

273:             end else if (!yellow_food_valid) begin

274:                 select_food_type = FOOD_YELLOW;

275:             end else begin

276:                 select_food_type = FOOD_GREEN;

277:             end

278:         end

279:     endfunction

280:

281:     function all_food_present;

282:         input unused;

283:         begin

284:             all_food_present = food_valid && yellow_food_valid && green_food_valid;

select_food_type 还做了兜底：如果按权重选中的那种食物已经存在，就优先改选当前未出现的类型。all_food_present 为真时不再生成，避免三种食物重复铺满。

食物生成请求、随机起点与空格扫描（vga_snake.srcs/sources_1/new/snake_game.v:714-739）

714:                 if (pending_food) begin

715:                     if (!food_type_available(pending_food_type) || food_scan_idx >= GRID_CELLS) begin

716:                         pending_food <= 1'b0;

717:                     end else if ((food_scan_idx != head_idx) &&

718:                                  !snake_map[food_scan_idx] &&

719:                                  !food_occupies_index(food_scan_idx) &&

720:                                  !enemy_occupies_index(food_scan_idx)) begin

721:                         case (pending_food_type)

722:                             FOOD_RED: begin

723:                                 food_idx <= food_scan_idx;

724:                                 food_valid <= 1'b1;

725:                             end

726:                             FOOD_YELLOW: begin

727:                                 yellow_food_idx <= food_scan_idx;

728:                                 yellow_food_valid <= 1'b1;

729:                             end

730:                             FOOD_GREEN: begin

731:                                 green_food_idx <= food_scan_idx;

732:                                 green_food_valid <= 1'b1;

733:                             end

734:                         endcase

735:                         pending_food <= 1'b0;

736:                         food_spawn_countdown <= next_food_delay(lfsr ^ food_scan_idx);

737:                     end else begin

738:                         food_scan_idx <= food_scan_idx + 1'b1;

739:                     end

运行中按倒计时触发食物生成（vga_snake.srcs/sources_1/new/snake_game.v:1232-1240）

1232:                             if (all_food_present(1'b0)) begin

1233:                                 pending_food <= 1'b0;

1234:                                 food_spawn_countdown <= next_food_delay(lfsr ^ 16'h55AA);

1235:                             end else if (!pending_food) begin

1236:                                 if (food_spawn_countdown == 0) begin

1237:                                     pending_food_type <= select_food_type(lfsr ^ head_idx);

1238:                                     pending_food <= 1'b1;

1239:                                     food_scan_idx <= lfsr % GRID_CELLS;

1240:                                 end else begin

食物效果在玩家移动到 nidx 后判断：红色加 1 分并立即增长 1 节；黄色加 2 分并设置 growth_pending，使后续再额外长一节；蓝色生命食物不增长身体，而是在 lives < MAX_LIVES 时加生命。

吃到不同食物后的效果（vga_snake.srcs/sources_1/new/snake_game.v:1107-1165）

1107:                                 if (food_valid && (nidx == food_idx)) begin

1108:                                     player_ate_food = 1'b1;

1109:                                     eaten_food_type = FOOD_RED;

1110:                                 end else if (yellow_food_valid && (nidx == yellow_food_idx)) begin

1111:                                     player_ate_food = 1'b1;

1112:                                     eaten_food_type = FOOD_YELLOW;

1113:                                 end else if (green_food_valid && (nidx == green_food_idx)) begin

1114:                                     player_ate_food = 1'b1;

1115:                                     eaten_food_type = FOOD_GREEN;

1116:                                 end

1117:

1118:                                 if (player_ate_food && !player_hit_enemy_body && (eaten_food_type != FOOD_GREEN)) begin

1119:                                     if (slen < MAXLEN) begin

1120:                                         player_grew = 1'b1;

1121:                                         new_head_ptr = prev_snake_index(head_ptr);

1122:                                         snake_pos[new_head_ptr] <= nidx;

1123:                                         head_ptr <= new_head_ptr;

1124:                                         slen <= slen + 1;

1125:                                         snake_map[head_idx] <= 1'b1;

1126:                                         head_idx <= nidx;

1127:                                         hx <= nx;

1128:                                         hy <= ny;

1129:                                         cur_dir <= next_dir;

1130:                                         if (eaten_food_type == FOOD_RED) begin

1131:                                             score <= score + 1;

1132:                                             food_valid <= 1'b0;

1133:                                         end else begin

1134:                                             score <= score + 2;

1135:                                             yellow_food_valid <= 1'b0;

1136:                                             if (slen < MAXLEN - 1)

1137:                                                 growth_pending <= growth_pending + 1'b1;

1138:                                         end

1139:                                     end else begin

1140:                                         state <= 3;

1141:                                         player_gameover = 1'b1;

1142:                                     end

1143:                                 end else begin

1144:                                     new_head_ptr = prev_snake_index(head_ptr);

1145:                                     next_tail_ptr = prev_snake_index(tail_ptr);

1146:                                     snake_pos[new_head_ptr] <= nidx;

1147:                                     head_ptr <= new_head_ptr;

1148:                                     if (growth_pending != 0) begin

1149:                                         slen <= slen + 1'b1;

1150:                                         growth_pending <= growth_pending - 1'b1;

1151:                                     end else begin

1152:                                         tail_ptr <= next_tail_ptr;

1153:                                         snake_map[tailidx] <= 1'b0;

1154:                                     end

1155:                                     snake_map[head_idx] <= 1'b1;

1156:                                     head_idx <= nidx;

1157:                                     hx <= nx;

1158:                                     hy <= ny;

1159:                                     cur_dir <= next_dir;

1160:                                     if (player_hit_enemy_body)

1161:                                         score <= score + 5;

1162:                                     else if (player_ate_food && (eaten_food_type == FOOD_GREEN)) begin

1163:                                         green_food_valid <= 1'b0;

1164:                                         if (lives < MAX_LIVES)

1165:                                             lives <= lives + 1'b1;

5. 玩家身体变长和移动

玩家蛇的数据结构由两部分组成：snake_pos 环形数组记录从头到尾的格子序列，head_ptr/tail_ptr 指向头尾；snake_map 是位图，快速判断某个格子是否为身体。注意 snake_map 不包含当前头，只包含身体段。

蛇身存储结构（vga_snake.srcs/sources_1/new/snake_game.v:179-197）

179:     reg [GRID_CELLS-1:0] snake_map;

180:     reg [15:0] snake_pos [0:MAXLEN-1];

181:     reg [15:0] slen;

182:     reg [15:0] head_ptr;

183:     reg [15:0] tail_ptr;

184:     reg [15:0] head_idx;

185:     reg [15:0] food_idx;

186:     reg        food_valid;

187:     reg [15:0] yellow_food_idx;

188:     reg        yellow_food_valid;

189:     reg [15:0] green_food_idx;

190:     reg        green_food_valid;

191:     reg        pending_food;

192:     reg [1:0]  pending_food_type;

193:     reg [15:0] food_scan_idx;

194:     reg [15:0] food_spawn_countdown;

195:     reg [5:0]  invincible_ticks;

196:     reg        invincible_blink;

197:     reg [3:0]  growth_pending;

开局初始化蛇头、两节身体和方向（vga_snake.srcs/sources_1/new/snake_game.v:740-803）

740:                 end else if (start && state == 0) begin

741:                     hx <= COLS/2;

742:                     hy <= ROWS/2;

743:                     head_idx <= INIT_HEAD_IDX;

744:                     slen <= 3;

745:                     head_ptr <= 0;

746:                     tail_ptr <= 2;

747:                     snake_pos[0] <= INIT_HEAD_IDX;

748:                     snake_pos[1] <= INIT_BODY1_IDX;

749:                     snake_pos[2] <= INIT_BODY2_IDX;

750:                     snake_map <= initial_snake_map(1'b0);

751:

752:                     fx = lfsr % COLS;

753:                     fy = (lfsr >> 4) % ROWS;

754:                     idx = fy * COLS + fx;

755:                     food_valid <= 1'b0;

756:                     yellow_food_valid <= 1'b0;

757:                     green_food_valid <= 1'b0;

758:                     pending_food_type <= select_food_type(lfsr);

759:                     if (!is_initial_snake_index(idx)) begin

760:                         case (select_food_type(lfsr))

761:                             FOOD_RED: begin

762:                                 food_idx <= idx;

763:                                 food_valid <= 1'b1;

764:                             end

765:                             FOOD_YELLOW: begin

766:                                 yellow_food_idx <= idx;

767:                                 yellow_food_valid <= 1'b1;

768:                             end

769:                             FOOD_GREEN: begin

770:                                 green_food_idx <= idx;

771:                                 green_food_valid <= 1'b1;

772:                             end

773:                         endcase

774:                     end else begin

775:                         pending_food <= 1'b1;

776:                         food_scan_idx <= 0;

777:                     end

778:

779:                     state <= 1;

780:                     score <= 0;

781:                     lives <= INITIAL_LIVES;

782:                     invincible_ticks <= 0;

783:                     invincible_blink <= 1'b1;

784:                     growth_pending <= 0;

785:                     pending_player_respawn <= 1'b0;

786:                     respawn_scan_idx <= lfsr % GRID_CELLS;

787:                     respawn_path_mode <= lfsr[3:2];

788:                     respawn_phase <= RESP_SCAN;

789:                     respawn_build_idx <= 0;

790:                     respawn_head_idx <= 0;

791:                     respawn_forward_idx <= 0;

792:                     respawn_head_dir <= DIR_RIGHT;

793:                     respawn_scan_x <= random_x(lfsr);

794:                     respawn_scan_y <= random_y(lfsr);

795:                     respawn_build_x <= 0;

796:                     respawn_build_y <= 0;

797:                     respawn_build_dir <= DIR_LEFT;

798:                     respawn_bend_seen <= 1'b0;

799:                     respawn_last_dir <= DIR_LEFT;

800:                     food_spawn_countdown <= next_food_delay(lfsr ^ 16'h2468);

801:                     cur_dir <= start_direction(dir, dir_valid);

802:                     next_dir <= start_direction(dir, dir_valid);

803:                     enemy_spawn_countdown <= next_spawn_delay(lfsr ^ 16'h1357);

每个 tick 先根据 next_dir 算出下一格 nx/ny，再换算为 nidx。普通移动时：插入新头，旧头写入 snake_map，尾巴从 snake_map 清掉并移动 tail_ptr。吃红/黄食物时：插入新头但不删尾巴，所以 slen 增加。

根据方向计算下一格（vga_snake.srcs/sources_1/new/snake_game.v:1009-1032）

1009:                     if (tick && state == 1 && !pause && !pending_player_respawn) begin

1010:                         player_gameover = 1'b0;

1011:                         player_grew = 1'b0;

1012:                         player_hit_enemy_body = 1'b0;

1013:                         player_ate_food = 1'b0;

1014:                         eaten_food_type = FOOD_RED;

1015:                         player_damaged = 1'b0;

1016:                         killed_enemy_slot = 3'd0;

1017:

1018:                         if (invincible_ticks != 0) begin

1019:                             invincible_ticks <= invincible_ticks - 1'b1;

1020:                             invincible_blink <= ~invincible_blink;

1021:                         end else begin

1022:                             invincible_blink <= 1'b1;

1023:                         end

1024:

1025:                         nx = hx;

1026:                         ny = hy;

1027:                         case (next_dir)

1028:                             DIR_UP:    ny = hy - 1;

1029:                             DIR_DOWN:  ny = hy + 1;

1030:                             DIR_LEFT:  nx = hx - 1;

1031:                             DIR_RIGHT: nx = hx + 1;

1032:                         endcase

吃食物时立即增长（vga_snake.srcs/sources_1/new/snake_game.v:1118-1138）

1118:                                 if (player_ate_food && !player_hit_enemy_body && (eaten_food_type != FOOD_GREEN)) begin

1119:                                     if (slen < MAXLEN) begin

1120:                                         player_grew = 1'b1;

1121:                                         new_head_ptr = prev_snake_index(head_ptr);

1122:                                         snake_pos[new_head_ptr] <= nidx;

1123:                                         head_ptr <= new_head_ptr;

1124:                                         slen <= slen + 1;

1125:                                         snake_map[head_idx] <= 1'b1;

1126:                                         head_idx <= nidx;

1127:                                         hx <= nx;

1128:                                         hy <= ny;

1129:                                         cur_dir <= next_dir;

1130:                                         if (eaten_food_type == FOOD_RED) begin

1131:                                             score <= score + 1;

1132:                                             food_valid <= 1'b0;

1133:                                         end else begin

1134:                                             score <= score + 2;

1135:                                             yellow_food_valid <= 1'b0;

1136:                                             if (slen < MAXLEN - 1)

1137:                                                 growth_pending <= growth_pending + 1'b1;

1138:                                         end

普通移动或延迟增长（vga_snake.srcs/sources_1/new/snake_game.v:1143-1159）

1143:                                 end else begin

1144:                                     new_head_ptr = prev_snake_index(head_ptr);

1145:                                     next_tail_ptr = prev_snake_index(tail_ptr);

1146:                                     snake_pos[new_head_ptr] <= nidx;

1147:                                     head_ptr <= new_head_ptr;

1148:                                     if (growth_pending != 0) begin

1149:                                         slen <= slen + 1'b1;

1150:                                         growth_pending <= growth_pending - 1'b1;

1151:                                     end else begin

1152:                                         tail_ptr <= next_tail_ptr;

1153:                                         snake_map[tailidx] <= 1'b0;

1154:                                     end

1155:                                     snake_map[head_idx] <= 1'b1;

1156:                                     head_idx <= nidx;

1157:                                     hx <= nx;

1158:                                     hy <= ny;

1159:                                     cur_dir <= next_dir;

关键点：普通移动允许蛇头走到当前尾巴的位置，因为尾巴在同一 tick 会离开，所以自撞判断写成 snake_map[nidx] && nidx != tailidx。

6. 游戏结束后 GAMEOVER 的显示

游戏结束由 snake_game 把 state 置为 3。top.v 中为 GAMEOVER 定义了 5x7 点阵字体：gameover_letter_pixel 根据字符编号和字体内坐标返回该像素是否点亮，gameover_pixel 再把 G A M E O V E R 八个字符排列起来。

GAMEOVER 字体和字符排布函数（vga_snake.srcs/sources_1/new/top.v:110-234）

110:     function gameover_letter_pixel;

111:         input [3:0] ch;

112:         input [2:0] fx;

113:         input [2:0] fy;

114:         reg [4:0] row;

115:         begin

116:             row = 5'b00000;

117:             case (ch)

118:                 4'd0: begin // G

119:                     case (fy)

120:                         3'd0: row = 5'b11111;

121:                         3'd1: row = 5'b10000;

122:                         3'd2: row = 5'b10000;

123:                         3'd3: row = 5'b10111;

124:                         3'd4: row = 5'b10001;

125:                         3'd5: row = 5'b10001;

126:                         3'd6: row = 5'b11111;

127:                     endcase

128:                 end

129:                 4'd1: begin // A

130:                     case (fy)

131:                         3'd0: row = 5'b01110;

132:                         3'd1: row = 5'b10001;

133:                         3'd2: row = 5'b10001;

134:                         3'd3: row = 5'b11111;

135:                         3'd4: row = 5'b10001;

136:                         3'd5: row = 5'b10001;

137:                         3'd6: row = 5'b10001;

138:                     endcase

139:                 end

140:                 4'd2: begin // M

141:                     case (fy)

142:                         3'd0: row = 5'b10001;

143:                         3'd1: row = 5'b11011;

144:                         3'd2: row = 5'b10101;

145:                         3'd3: row = 5'b10101;

146:                         3'd4: row = 5'b10001;

147:                         3'd5: row = 5'b10001;

148:                         3'd6: row = 5'b10001;

149:                     endcase

150:                 end

151:                 4'd3: begin // E

152:                     case (fy)

153:                         3'd0: row = 5'b11111;

154:                         3'd1: row = 5'b10000;

155:                         3'd2: row = 5'b10000;

156:                         3'd3: row = 5'b11110;

157:                         3'd4: row = 5'b10000;

158:                         3'd5: row = 5'b10000;

159:                         3'd6: row = 5'b11111;

160:                     endcase

161:                 end

162:                 4'd4: begin // O

163:                     case (fy)

164:                         3'd0: row = 5'b01110;

165:                         3'd1: row = 5'b10001;

166:                         3'd2: row = 5'b10001;

167:                         3'd3: row = 5'b10001;

168:                         3'd4: row = 5'b10001;

169:                         3'd5: row = 5'b10001;

170:                         3'd6: row = 5'b01110;

171:                     endcase

172:                 end

173:                 4'd5: begin // V

174:                     case (fy)

175:                         3'd0: row = 5'b10001;

176:                         3'd1: row = 5'b10001;

177:                         3'd2: row = 5'b10001;

178:                         3'd3: row = 5'b10001;

179:                         3'd4: row = 5'b10001;

180:                         3'd5: row = 5'b01010;

181:                         3'd6: row = 5'b00100;

182:                     endcase

183:                 end

184:                 default: begin // R

185:                     case (fy)

186:                         3'd0: row = 5'b11110;

187:                         3'd1: row = 5'b10001;

188:                         3'd2: row = 5'b10001;

189:                         3'd3: row = 5'b11110;

190:                         3'd4: row = 5'b10100;

191:                         3'd5: row = 5'b10010;

192:                         3'd6: row = 5'b10001;

193:                     endcase

194:                 end

195:             endcase

196:

197:             case (fx)

198:                 3'd0: gameover_letter_pixel = row[4];

199:                 3'd1: gameover_letter_pixel = row[3];

200:                 3'd2: gameover_letter_pixel = row[2];

201:                 3'd3: gameover_letter_pixel = row[1];

202:                 3'd4: gameover_letter_pixel = row[0];

203:                 default: gameover_letter_pixel = 1'b0;

204:             endcase

205:         end

206:     endfunction

207:

208:     function gameover_pixel;

209:         input [5:0] tx;

210:         input [2:0] ty;

211:         reg [3:0] ch;

212:         reg [2:0] fx;

213:         begin

214:             ch = 4'd15;

215:             fx = 3'd7;

216:             if (tx < 6'd5) begin

217:                 ch = 4'd0; fx = tx[2:0];

218:             end else if ((tx >= 6'd6) && (tx < 6'd11)) begin

219:                 ch = 4'd1; fx = tx - 6'd6;

220:             end else if ((tx >= 6'd12) && (tx < 6'd17)) begin

221:                 ch = 4'd2; fx = tx - 6'd12;

222:             end else if ((tx >= 6'd18) && (tx < 6'd23)) begin

223:                 ch = 4'd3; fx = tx - 6'd18;

224:             end else if ((tx >= 6'd24) && (tx < 6'd29)) begin

225:                 ch = 4'd4; fx = tx - 6'd24;

226:             end else if ((tx >= 6'd30) && (tx < 6'd35)) begin

227:                 ch = 4'd5; fx = tx - 6'd30;

228:             end else if ((tx >= 6'd36) && (tx < 6'd41)) begin

229:                 ch = 4'd3; fx = tx - 6'd36;

230:             end else if ((tx >= 6'd42) && (tx < 6'd47)) begin

231:                 ch = 4'd6; fx = tx - 6'd42;

232:             end

233:             gameover_pixel = (ch != 4'd15) && gameover_letter_pixel(ch, fx, ty);

234:         end

最终输出时，如果 state==3，top.v 不再使用 grid_renderer 的游戏画面颜色，而是把背景清黑，只在 GAMEOVER 点阵所在像素输出白色。

GAMEOVER 覆盖 VGA 输出（vga_snake.srcs/sources_1/new/top.v:508-525）

508:     // GAMEOVER clears the game field and draws only pixel text.

509:     localparam GAMEOVER_TEXT_X = 10'd132;

510:     localparam GAMEOVER_TEXT_Y = 9'd212;

511:     localparam GAMEOVER_TEXT_W = 10'd376;

512:     localparam GAMEOVER_TEXT_H = 9'd56;

513:

514:     wire gameover_box =

515:         (pixel_x >= GAMEOVER_TEXT_X) && (pixel_x < GAMEOVER_TEXT_X + GAMEOVER_TEXT_W) &&

516:         (pixel_y >= GAMEOVER_TEXT_Y) && (pixel_y < GAMEOVER_TEXT_Y + GAMEOVER_TEXT_H);

517:     wire [5:0] gameover_text_x = (pixel_x - GAMEOVER_TEXT_X) >> 3;

518:     wire [2:0] gameover_text_y = (pixel_y - GAMEOVER_TEXT_Y) >> 3;

519:     wire gameover_text_on =

520:         active && (state == 2'd3) && gameover_box &&

521:         gameover_pixel(gameover_text_x, gameover_text_y);

522:

523:     assign vga_r = (state == 2'd3) ? (gameover_text_on ? 3'b111 : 3'b000) : rgb_r;

524:     assign vga_g = (state == 2'd3) ? (gameover_text_on ? 3'b111 : 3'b000) : rgb_g;

525:     assign vga_b = (state == 2'd3) ? (gameover_text_on ? 3'b111 : 3'b000) : rgb_b;

7. 画面中颜色的控制

颜色控制分两步。第一步在 snake_game.v 中按优先级把当前 cell_x/cell_y 转换为 cell_state：玩家头、敌人头、食物、敌人身体、玩家身体、空格。第二步在 grid_renderer.v 中把 cell_state 映射成 3 位 R/G/B。

格子状态优先级输出（vga_snake.srcs/sources_1/new/snake_game.v:571-600）

571:     wire [15:0] cell_index_raw = cell_y * COLS + cell_x;

572:     wire [15:0] cell_index = cell_in_range ? cell_index_raw : 16'd0;

573:     wire        show_cells = cell_in_range && (state != 2'd0);

574:     wire        show_player = !(invincible_ticks != 0 && !invincible_blink);

575:

576:     always @(*) begin

577:         enemy_head_cell = 1'b0;

578:         enemy_body_cell = 1'b0;

579:         for (de = 0; de < MAX_ENEMIES; de = de + 1) begin

580:             display_base = de * ENEMY_MAX_LEN;

581:             if (enemy_alive[de]) begin

582:                 if (enemy_pos[display_base] == cell_index)

583:                     enemy_head_cell = 1'b1;

584:                 for (ds = 1; ds < ENEMY_MAX_LEN; ds = ds + 1) begin

585:                     if ((ds < enemy_len[de]) && (enemy_pos[display_base + ds] == cell_index))

586:                         enemy_body_cell = 1'b1;

587:                 end

588:             end

589:         end

590:     end

591:

592:     assign cell_state =

593:         (!show_cells) ? EMPTY :

594:         (show_player && (cell_index == head_idx)) ? HEAD :

595:         enemy_head_cell ? ENEMY_HEAD :

596:         (food_valid && (cell_index == food_idx)) ? RED_FOOD :

597:         (yellow_food_valid && (cell_index == yellow_food_idx)) ? YELLOW_FOOD :

598:         (green_food_valid && (cell_index == green_food_idx)) ? BLUE_FOOD :

599:         enemy_body_cell ? ENEMY_BODY :

600:         (show_player && snake_map[cell_index]) ? BODY :

cell_state 到 RGB 的颜色表（vga_snake.srcs/sources_1/new/grid_renderer.v:42-91）

42:     // compute cell coordinates by division by CELL_W/H (powers of two assumed)

43:     // CELL_W=16 -> >>4

44:     assign cell_x = pixel_x[9:4]; // pixel_x / 16

45:     assign cell_y = pixel_y[8:4]; // pixel_y / 16

46:

47:     // Optionally draw cell borders: thin grid lines

48:     wire [3:0] local_x = pixel_x[3:0]; // within cell 0..15

49:     wire [3:0] local_y = pixel_y[3:0];

50:     wire border = (local_x == 0) || (local_y == 0);

51:

52:     always @(posedge clk_pix or negedge rst_n) begin

53:         if (!rst_n) begin

54:             rgb_r <= 3'b000; rgb_g <= 3'b000; rgb_b <= 3'b000;

55:         end else begin

56:             if (!active) begin

57:                 rgb_r <= 3'b000; rgb_g <= 3'b000; rgb_b <= 3'b000;

58:             end else begin

59:                 case (cell_state)

60:                     3'b000: begin // empty

61:                         if (border) begin

62:                             rgb_r <= 3'b010; rgb_g <= 3'b010; rgb_b <= 3'b010; // light grey grid line

63:                         end else begin

64:                             rgb_r <= 3'b000; rgb_g <= 3'b000; rgb_b <= 3'b000; // black background

65:                         end

66:                     end

67:                     3'b001: begin // player body -> green

68:                         rgb_r <= 3'b000; rgb_g <= 3'b111; rgb_b <= 3'b000;

69:                     end

70:                     3'b010: begin // player head -> white

71:                         rgb_r <= 3'b111; rgb_g <= 3'b111; rgb_b <= 3'b111;

72:                     end

73:                     3'b011: begin // red food

74:                         rgb_r <= 3'b111; rgb_g <= 3'b000; rgb_b <= 3'b000;

75:                     end

76:                     3'b100: begin // yellow food

77:                         rgb_r <= 3'b111; rgb_g <= 3'b111; rgb_b <= 3'b000;

78:                     end

79:                     3'b101: begin // blue life food

80:                         rgb_r <= 3'b000; rgb_g <= 3'b000; rgb_b <= 3'b111;

81:                     end

82:                     3'b110: begin // enemy head -> magenta

83:                         rgb_r <= 3'b111; rgb_g <= 3'b000; rgb_b <= 3'b111;

84:                     end

85:                     3'b111: begin // enemy body -> dark magenta

86:                         rgb_r <= 3'b011; rgb_g <= 3'b000; rgb_b <= 3'b011;

87:                     end

88:                     default: begin

89:                         rgb_r <= 3'b000; rgb_g <= 3'b000; rgb_b <= 3'b000;

90:                     end

91:                 endcase

| cell_state | 含义 | RGB | 显示颜色 |
| --- | --- | --- | --- |
| 000 | 空格 | 000/000/000，边框 010/010/010 | 黑色背景，灰色网格线 |
| 001 | 玩家身体 | 000/111/000 | 绿色 |
| 010 | 玩家头 | 111/111/111 | 白色 |
| 011 | 红色食物 | 111/000/000 | 红色 |
| 100 | 黄色食物 | 111/111/000 | 黄色 |
| 101 | 生命食物 | 000/000/111 | 蓝色 |
| 110 | 敌人头 | 111/000/111 | 品红 |
| 111 | 敌人身体 | 011/000/011 | 暗品红 |

分析：grid_renderer 在 active_video 为 0 时输出黑色，避免消隐区出现无效颜色；GAMEOVER 状态又在 top.v 末尾覆盖 RGB，因此 GAMEOVER 的显示优先级最高。

8. 信号如何传输到显示屏

显示链路为：sys_clock 进入 Vivado block design 的 Clocking Wizard，产生 clk_pix；top.v 中 vga_timing 用 clk_pix 产生 hsync/vsync/active/pixel_x/pixel_y；grid_renderer 根据像素坐标查询 snake_game 的 cell_state 并输出 rgb_r/g/b；top.v 最终把 rgb 或 GAMEOVER 文字输出到 vga_r/g/b；board_top 和 XDC 再把这些逻辑端口连到实际 VGA 管脚。

VGA 640x480 时序参数与同步信号（vga_snake.srcs/sources_1/new/vga_timing.v:34-94）

34:     localparam H_VISIBLE = 640;

35:     localparam H_FRONT   = 16;

36:     localparam H_SYNC    = 96;

37:     localparam H_BACK    = 48;

38:     localparam H_TOTAL   = H_VISIBLE + H_FRONT + H_SYNC + H_BACK; //800

39:

40:     localparam V_VISIBLE = 480;

41:     localparam V_FRONT   = 10;

42:     localparam V_SYNC    = 2;

43:     localparam V_BACK    = 33;

44:     localparam V_TOTAL   = V_VISIBLE + V_FRONT + V_SYNC + V_BACK; //525

45:

46:     // counters

47:     reg [9:0] hcnt;

48:     reg [9:0] vcnt; // needs to count to 524

49:

50:     always @(posedge clk_pix or negedge rst_n) begin

51:         if (!rst_n) begin

52:             hcnt <= 0;

53:             vcnt <= 0;

54:             hsync <= 1;

55:             vsync <= 1;

56:             pixel_x <= 0;

57:             pixel_y <= 0;

58:         end else begin

59:             if (hcnt == H_TOTAL - 1) begin

60:                 hcnt <= 0;

61:                 if (vcnt == V_TOTAL - 1)

62:                     vcnt <= 0;

63:                 else

64:                     vcnt <= vcnt + 1;

65:             end else begin

66:                 hcnt <= hcnt + 1;

67:             end

68:

69:             // HS: active during sync pulse (standard active low)

70:             if (hcnt >= (H_VISIBLE + H_FRONT) && hcnt < (H_VISIBLE + H_FRONT + H_SYNC))

71:                 hsync <= 0;

72:             else

73:                 hsync <= 1;

74:

75:             // VS

76:             if (vcnt >= (V_VISIBLE + V_FRONT) && vcnt < (V_VISIBLE + V_FRONT + V_SYNC))

77:                 vsync <= 0;

78:             else

79:                 vsync <= 1;

80:

81:             // pixel coordinates within visible area

82:             if (hcnt < H_VISIBLE)

83:                 pixel_x <= hcnt;

84:             else

85:                 pixel_x <= 0;

86:

87:             if (vcnt < V_VISIBLE)

88:                 pixel_y <= vcnt;

89:             else

90:                 pixel_y <= 0;

91:         end

92:     end

93:

94:     assign active_video = (hcnt < H_VISIBLE) && (vcnt < V_VISIBLE);

top 中连接时序、游戏和渲染模块（vga_snake.srcs/sources_1/new/top.v:310-342）

310:     vga_timing u_vga_timing (

311:         .clk_pix(clk_pix),

312:         .rst_n(rst_n),

313:         .hsync(hsync),

314:         .vsync(vsync),

315:         .active_video(active),

316:         .pixel_x(pixel_x),

317:         .pixel_y(pixel_y)

318:     );

319:

320:     //==================================================

321:     // slow tick generator (for snake move)

322:     // uses clk_pix domain

323:     //==================================================

324:     reg [31:0] tick_cnt;

325:     reg tick_q;

326:

327:     always @(posedge clk_pix or negedge rst_n) begin

328:         if (!rst_n) begin

329:             tick_cnt <= 0;

330:             tick_q <= 1'b0;

331:         end else begin

332:             if (tick_cnt >= (TICK_DIV - 1)) begin

333:                 tick_cnt <= 0;

334:                 tick_q <= 1'b1;

335:             end else begin

336:                 tick_cnt <= tick_cnt + 1;

337:                 tick_q <= 1'b0;

338:             end

339:         end

340:     end

341:

342:     wire tick = tick_q;

snake_game 与 grid_renderer 的端口连接（vga_snake.srcs/sources_1/new/top.v:457-506）

457:     snake_game #(

458:         .COLS(40),

459:         .ROWS(30),

460:         .MAXLEN(128),

461:         .MAX_ENEMIES(5),

462:         .ENEMY_MIN_LEN(ENEMY_MIN_LEN),

463:         .ENEMY_MAX_LEN(ENEMY_MAX_LEN),

464:         .ENEMY_SPAWN_MIN_TICKS(ENEMY_SPAWN_MIN_TICKS),

465:         .ENEMY_SPAWN_RANDOM_MASK(ENEMY_SPAWN_RANDOM_MASK)

466:     ) u_snake (

467:         .clk(clk_pix),

468:         .rst_n(rst_n),

469:         .dir(dir_req),

470:         .dir_valid(dir_req_valid),

471:         .start(start),

472:         .pause(pause),

473:         .reset_game(reset_game),

474:         .tick(tick),

475:         .enemy_limit(active_enemy_limit),

476:         .cell_x(cell_x),

477:         .cell_y(cell_y),

478:         .cell_state(cell_status),

479:         .score(score),

480:         .lives(lives),

481:         .state(state)

482:     );

483:

484:     assign gpio_status = {lives, status_state, score};

485:

486:     //==================================================

487:     // grid_renderer instantiation

488:     //==================================================

489:     wire [2:0] rgb_r, rgb_g, rgb_b;

490:

491:     grid_renderer #(

492:         .CELL_W(16),

493:         .CELL_H(16)

494:     ) u_renderer (

495:         .clk_pix(clk_pix),

496:         .rst_n(rst_n),

497:         .active(active),

498:         .pixel_x(pixel_x),

499:         .pixel_y(pixel_y),

500:         .cell_x(cell_x),

501:         .cell_y(cell_y),

502:         .cell_state(cell_status),

503:         .rgb_r(rgb_r),

504:         .rgb_g(rgb_g),

505:         .rgb_b(rgb_b)

506:     );

板级封装把 VGA 信号接到 wrapper（vga_snake.srcs/sources_1/new/board_top.v:3-35）

3: module board_top #(

4:     parameter ONE_SECOND_CLKS = 100000000,

5:     parameter SEVENSEG_REFRESH_DIV = 100000

6: )(

7:     input  wire       sys_clock,

8:     input  wire       reset,

9:     input  wire [4:0] gpio_btn,

10:     input  wire       usb_uart_rxd,

11:     output wire       usb_uart_txd,

12:     output wire       hsync,

13:     output wire       vsync,

14:     output wire [2:0] vga_r,

15:     output wire [2:0] vga_g,

16:     output wire [2:0] vga_b,

17:     output reg  [6:0] seg,

18:     output reg        dp,

19:     output reg  [7:0] an

20: );

21:

22:     wire [21:0] gpio_status_ext;

23:

24:     vga_snake_wrapper u_system (

25:         .gpio_btn(gpio_btn),

26:         .gpio_status_ext(gpio_status_ext),

27:         .hsync(hsync),

28:         .reset(reset),

29:         .sys_clock(sys_clock),

30:         .usb_uart_rxd(usb_uart_rxd),

31:         .usb_uart_txd(usb_uart_txd),

32:         .vga_b(vga_b),

33:         .vga_g(vga_g),

34:         .vga_r(vga_r),

35:         .vsync(vsync)

VGA 逻辑端口到 Nexys DDR4 管脚（vga_snake.srcs/constrs_1/new/nexys_ddr4_vga.xdc:4-41）

4: # Clock and reset

5: set_property PACKAGE_PIN E3 [get_ports sys_clock]

6: set_property IOSTANDARD LVCMOS33 [get_ports sys_clock]

7: create_clock -add -name sys_clk_pin -period 10.000 -waveform {0 5.000} [get_ports sys_clock]

8:

9: set_property PACKAGE_PIN C12 [get_ports reset]

10: set_property IOSTANDARD LVCMOS33 [get_ports reset]

11:

12: # VGA sync

13: set_property PACKAGE_PIN B11 [get_ports hsync]

14: set_property IOSTANDARD LVCMOS33 [get_ports hsync]

15:

16: set_property PACKAGE_PIN B12 [get_ports vsync]

17: set_property IOSTANDARD LVCMOS33 [get_ports vsync]

18:

19: # VGA red bits, using the first three pins from the provided four-bit list.

20: set_property PACKAGE_PIN A4 [get_ports {vga_r[2]}]

21: set_property IOSTANDARD LVCMOS33 [get_ports {vga_r[2]}]

22: set_property PACKAGE_PIN C5 [get_ports {vga_r[1]}]

23: set_property IOSTANDARD LVCMOS33 [get_ports {vga_r[1]}]

24: set_property PACKAGE_PIN B4 [get_ports {vga_r[0]}]

25: set_property IOSTANDARD LVCMOS33 [get_ports {vga_r[0]}]

26:

27: # VGA green bits, using the first three pins from the provided four-bit list.

28: set_property PACKAGE_PIN A6 [get_ports {vga_g[2]}]

29: set_property IOSTANDARD LVCMOS33 [get_ports {vga_g[2]}]

30: set_property PACKAGE_PIN B6 [get_ports {vga_g[1]}]

31: set_property IOSTANDARD LVCMOS33 [get_ports {vga_g[1]}]

32: set_property PACKAGE_PIN A5 [get_ports {vga_g[0]}]

33: set_property IOSTANDARD LVCMOS33 [get_ports {vga_g[0]}]

34:

35: # VGA blue bits, using the first three pins from the provided four-bit list.

36: set_property PACKAGE_PIN D8 [get_ports {vga_b[2]}]

37: set_property IOSTANDARD LVCMOS33 [get_ports {vga_b[2]}]

38: set_property PACKAGE_PIN D7 [get_ports {vga_b[1]}]

39: set_property IOSTANDARD LVCMOS33 [get_ports {vga_b[1]}]

40: set_property PACKAGE_PIN C7 [get_ports {vga_b[0]}]

41: set_property IOSTANDARD LVCMOS33 [get_ports {vga_b[0]}]

补充：综合后的 block design 中 top_0 的 clk_pix 连接到 clk_wiz_1_clk_out2，top_0_hsync/vga_r/vga_g/vga_b/vsync 再直接赋给外部端口，因此显示信号从游戏逻辑一路传到板卡 VGA 端口。

9. 碰撞判定

碰撞判定集中在玩家 tick 更新和敌人 tick 更新中。玩家先算下一格 nidx，再按顺序判断边界、自身、敌人头、敌人身体、食物；敌人移动时再判断是否撞到玩家。

玩家撞墙、自撞与生命处理（vga_snake.srcs/sources_1/new/snake_game.v:1034-1074）

1034:                         if (slen == 0 || slen > MAXLEN) begin

1035:                             state <= 3;

1036:                             player_gameover = 1'b1;

1037:                         end else if (nx >= COLS || ny >= ROWS) begin

1038:                             if (invincible_ticks == 0) begin

1039:                                 player_damaged = 1'b1;

1040:                                 if (lives <= 1) begin

1041:                                     lives <= 0;

1042:                                     state <= 3;

1043:                                     player_gameover = 1'b1;

1044:                                 end else begin

1045:                                     lives <= lives - 1'b1;

1046:                                     invincible_ticks <= INVINCIBLE_TICKS;

1047:                                     invincible_blink <= 1'b0;

1048:                                 end

1049:                             end

1050:                         end else begin

1051:                             nidx = ny * COLS + nx;

1052:                             tailidx = snake_pos[tail_ptr];

1053:

1054:                             if ((nidx == head_idx) || (snake_map[nidx] && nidx != tailidx)) begin

1055:                                 player_damaged = 1'b1;

1056:                                 if (lives <= 1) begin

1057:                                     lives <= 0;

1058:                                     state <= 3;

1059:                                     player_gameover = 1'b1;

1060:                                 end else begin

1061:                                     lives <= lives - 1'b1;

1062:                                     invincible_ticks <= INVINCIBLE_TICKS;

1063:                                     invincible_blink <= 1'b0;

1064:                                     growth_pending <= 0;

1065:                                     pending_player_respawn <= 1'b1;

1066:                                     respawn_scan_idx <= 0;

1067:                                     respawn_scan_x <= random_x(lfsr);

1068:                                     respawn_scan_y <= random_y(lfsr);

1069:                                     respawn_head_dir <= lfsr[1:0];

1070:                                     respawn_path_mode <= lfsr[3:2];

1071:                                     respawn_phase <= RESP_SCAN;

1072:                                     respawn_build_idx <= 0;

1073:                                     respawn_bend_seen <= 1'b0;

1074:                                 end

玩家撞敌人头、敌人身体和食物（vga_snake.srcs/sources_1/new/snake_game.v:1075-1116）

1075:                             end else if (enemy_head_at_index(nidx)) begin

1076:                                 if (invincible_ticks == 0) begin

1077:                                     player_damaged = 1'b1;

1078:                                     if (lives <= 1) begin

1079:                                         lives <= 0;

1080:                                         state <= 3;

1081:                                         player_gameover = 1'b1;

1082:                                     end else begin

1083:                                         lives <= lives - 1'b1;

1084:                                         invincible_ticks <= INVINCIBLE_TICKS;

1085:                                         invincible_blink <= 1'b0;

1086:                                     end

1087:                                 end

1088:                             end else begin

1089:                                 for (ei = 0; ei < MAX_ENEMIES; ei = ei + 1) begin

1090:                                     enemy_base = ei * ENEMY_MAX_LEN;

1091:                                     if (enemy_alive[ei]) begin

1092:                                         for (ej = 1; ej < ENEMY_MAX_LEN; ej = ej + 1) begin

1093:                                             if ((ej < enemy_len[ei]) &&

1094:                                                 (enemy_pos[enemy_base + ej] == nidx) &&

1095:                                                 !player_hit_enemy_body) begin

1096:                                                 player_hit_enemy_body = 1'b1;

1097:                                                 killed_enemy_slot = ei[2:0];

1098:                                             end

1099:                                         end

1100:                                     end

1101:                                 end

1102:

1103:                                 if (player_hit_enemy_body) begin

1104:                                     enemy_alive[killed_enemy_slot] <= 1'b0;

1105:                                 end

1106:

1107:                                 if (food_valid && (nidx == food_idx)) begin

1108:                                     player_ate_food = 1'b1;

1109:                                     eaten_food_type = FOOD_RED;

1110:                                 end else if (yellow_food_valid && (nidx == yellow_food_idx)) begin

1111:                                     player_ate_food = 1'b1;

1112:                                     eaten_food_type = FOOD_YELLOW;

1113:                                 end else if (green_food_valid && (nidx == green_food_idx)) begin

1114:                                     player_ate_food = 1'b1;

1115:                                     eaten_food_type = FOOD_GREEN;

1116:                                 end

敌人移动撞玩家（vga_snake.srcs/sources_1/new/snake_game.v:1189-1208）

1189:                                     enemy_next_idx = next_grid_index(enemy_pos[enemy_base], try_dir);

1190:                                     enemy_can_move = enemy_move_valid(ei[2:0], try_dir);

1191:                                     if (enemy_can_move) begin

1192:                                         enemy_hit_player =

1193:                                             (enemy_next_idx == nidx) ||

1194:                                             (enemy_next_idx == head_idx) ||

1195:                                             (snake_map[enemy_next_idx] &&

1196:                                              (player_grew || (enemy_next_idx != tailidx)));

1197:

1198:                                         if (enemy_hit_player && (invincible_ticks == 0)) begin

1199:                                             if (lives <= 1) begin

1200:                                                 lives <= 0;

1201:                                                 state <= 3;

1202:                                                 player_gameover = 1'b1;

1203:                                             end else begin

1204:                                                 lives <= lives - 1'b1;

1205:                                                 invincible_ticks <= INVINCIBLE_TICKS;

1206:                                                 invincible_blink <= 1'b0;

1207:                                                 player_damaged = 1'b1;

1208:                                             end

| 碰撞类型 | 判定条件 | 结果 |
| --- | --- | --- |
| 撞墙 | nx>=COLS 或 ny>=ROWS | 无敌期外扣生命；生命为 1 时 state<=3。 |
| 自撞 | nidx==head_idx 或 snake_map[nidx] 且 nidx!=tailidx | 扣生命或 GAMEOVER；若还有生命则进入重生流程。 |
| 撞敌人头 | enemy_head_at_index(nidx) | 玩家受伤，生命不足时 GAMEOVER。 |
| 撞敌人身体 | enemy_pos[enemy_base+ej]==nidx，ej>=1 | 击杀该敌人，后续加 5 分。 |
| 吃食物 | nidx 等于 food_idx/yellow_food_idx/green_food_idx | 红/黄增长和加分；蓝色增加生命。 |
| 敌人撞玩家 | enemy_next_idx==nidx/head_idx 或撞 snake_map | 玩家受伤或 GAMEOVER。 |
| 生成冲突 | spawn_cell_free、food_occupies_index、enemy_occupies_index | 不在被占格生成，继续扫描下一个格子。 |

分析：碰撞检测把“玩家本 tick 的新头 nidx”和“旧头 head_idx”“身体位图 snake_map”“尾巴 tailidx”同时考虑，解决了蛇类游戏里常见的尾巴同 tick 移走问题。

10. 其他具体功能例子

10.1 按键同步、去抖、方向优先级、暂停/复位

top.v 对五个按键先做两级同步，再用 BUTTON_DEBOUNCE_CLKS 计数去抖。方向键只在上升沿产生一次 dir_req_valid；中键在运行时切换 pause，在 GAMEOVER 时产生 reset_game。

按键事件、去抖和暂停复位（vga_snake.srcs/sources_1/new/top.v:79-89）

79:     wire [4:0] btn_rise = btn_stable & ~btn_stable_d;

80:     wire btn_center_event = btn_rise[0];

81:     wire btn_up_event     = btn_rise[1];

82:     wire btn_left_event   = btn_rise[2];

83:     wire btn_right_event  = btn_rise[3];

84:     wire btn_down_event   = btn_rise[4];

85:

86:     wire start = (state == 2'd0) ? dir_req_valid : 1'b0;

87:     wire pause  = pause_reg;

88:     wire reset_game = reset_game_reg;

89:     wire [1:0] status_state = (pause && (state == 2'd1)) ? 2'd2 : state;

方向优先级与去抖（vga_snake.srcs/sources_1/new/top.v:236-304）

236:

237:     // 多键同时产生上升沿时使用固定优先级，正常单键操作不受影响。

238:     always @(*) begin

239:         dir_req = DIR_RIGHT;

240:         dir_req_valid = 1'b0;

241:

242:         if (btn_up_event) begin

243:             dir_req = DIR_UP;

244:             dir_req_valid = 1'b1;

245:         end else if (btn_down_event) begin

246:             dir_req = DIR_DOWN;

247:             dir_req_valid = 1'b1;

248:         end else if (btn_left_event) begin

249:             dir_req = DIR_LEFT;

250:             dir_req_valid = 1'b1;

251:         end else if (btn_right_event) begin

252:             dir_req = DIR_RIGHT;

253:             dir_req_valid = 1'b1;

254:         end

255:     end

256:

257:     always @(posedge clk_pix or negedge rst_n) begin

258:         if (!rst_n) begin

259:             btn_sync0 <= 5'b00000;

260:             btn_sync1 <= 5'b00000;

261:             btn_stable <= 5'b00000;

262:             btn_stable_d <= 5'b00000;

263:             btn_debounce_cnt <= 32'd0;

264:         end else begin

265:             btn_sync0 <= gpio_btn;

266:             btn_sync1 <= btn_sync0;

267:             btn_stable_d <= btn_stable;

268:

269:             if (btn_sync1 == btn_stable) begin

270:                 btn_debounce_cnt <= 32'd0;

271:             end else if (btn_debounce_cnt >= (BUTTON_DEBOUNCE_CLKS - 1)) begin

272:                 btn_stable <= btn_sync1;

273:                 btn_debounce_cnt <= 32'd0;

274:             end else begin

275:                 btn_debounce_cnt <= btn_debounce_cnt + 1'b1;

276:             end

277:         end

278:     end

279:

280:     // 中键：运行时暂停/继续，游戏结束时复位；空闲时不改变暂停状态。

281:     always @(posedge clk_pix or negedge rst_n) begin

282:         if (!rst_n) begin

283:             pause_reg <= 1'b0;

284:             reset_game_reg <= 1'b0;

285:         end else begin

286:             reset_game_reg <= 1'b0;

287:

288:             if (state == 2'd0) begin

289:                 pause_reg <= 1'b0;

290:             end

291:

292:             if (btn_center_event) begin

293:                 if (state == 2'd3) begin

294:                     reset_game_reg <= 1'b1;

295:                     pause_reg <= 1'b0;

296:                 end else if (state == 2'd1) begin

297:                     pause_reg <= ~pause_reg;

298:                 end

299:             end

300:         end

301:     end

302:

303:     //==================================================

304:     // VGA timing instantiation

10.2 移动 tick 控制速度

像素时钟是 25 MHz，但蛇不可能每个像素周期都移动。top.v 用 tick_cnt 分频，TICK_DIV=2500000 时约 10 Hz，即每秒移动约 10 格。

移动 tick 产生器（vga_snake.srcs/sources_1/new/top.v:320-342）

320:     //==================================================

321:     // slow tick generator (for snake move)

322:     // uses clk_pix domain

323:     //==================================================

324:     reg [31:0] tick_cnt;

325:     reg tick_q;

326:

327:     always @(posedge clk_pix or negedge rst_n) begin

328:         if (!rst_n) begin

329:             tick_cnt <= 0;

330:             tick_q <= 1'b0;

331:         end else begin

332:             if (tick_cnt >= (TICK_DIV - 1)) begin

333:                 tick_cnt <= 0;

334:                 tick_q <= 1'b1;

335:             end else begin

336:                 tick_cnt <= tick_cnt + 1;

337:                 tick_q <= 1'b0;

338:             end

339:         end

340:     end

341:

342:     wire tick = tick_q;

10.3 分数、生命、时间、敌人上限和数码管

score/lives/state 由 snake_game 输出；top.v 把生命、时间、分数拆成十进制位，通过 sevenseg_scan 轮流点亮 8 位数码管。同时 elapsed_min>=2 时 active_enemy_limit 从 3 增到 5，形成随时间提高难度的功能。

时间、分数拆位、敌人上限和数码管扫描（vga_snake.srcs/sources_1/new/top.v:347-447）

347:     // blank, score hundreds, score tens, score ones.

348:     //==================================================

349:     reg [31:0] one_second_cnt;

350:     reg [3:0] elapsed_min;

351:     reg [5:0] elapsed_sec;

352:     reg [31:0] sevenseg_refresh_cnt;

353:     reg [2:0] sevenseg_scan;

354:     reg [3:0] sevenseg_digit;

355:

356:     wire [9:0] score_capped = (score > 16'd999) ? 10'd999 : score[9:0];

357:     wire [3:0] score_hundreds = score_capped / 10'd100;

358:     wire [6:0] score_after_hundreds = score_capped - (score_hundreds * 7'd100);

359:     wire [3:0] score_tens = score_after_hundreds / 7'd10;

360:     wire [3:0] score_ones = score_after_hundreds - (score_tens * 4'd10);

361:     wire [3:0] display_lives = (lives > 4'd9) ? 4'd9 : lives;

362:     wire [3:0] display_min = elapsed_min;

363:     wire [3:0] display_sec_tens = elapsed_sec / 6'd10;

364:     wire [3:0] display_sec_ones = elapsed_sec - (display_sec_tens * 4'd10);

365:     wire [2:0] active_enemy_limit = (elapsed_min >= 4'd2) ? 3'd5 : 3'd3;

366:

367:     always @(posedge clk_pix or negedge rst_n) begin

368:         if (!rst_n) begin

369:             one_second_cnt <= 32'd0;

370:             elapsed_min <= 4'd0;

371:             elapsed_sec <= 6'd0;

372:         end else if ((state == 2'd0) || reset_game || start) begin

373:             one_second_cnt <= 32'd0;

374:             elapsed_min <= 4'd0;

375:             elapsed_sec <= 6'd0;

376:         end else if ((state == 2'd1) && !pause) begin

377:             if (one_second_cnt >= (ONE_SECOND_CLKS - 1)) begin

378:                 one_second_cnt <= 32'd0;

379:                 if (!((elapsed_min == 4'd9) && (elapsed_sec == 6'd59))) begin

380:                     if (elapsed_sec == 6'd59) begin

381:                         elapsed_sec <= 6'd0;

382:                         elapsed_min <= elapsed_min + 1'b1;

383:                     end else begin

384:                         elapsed_sec <= elapsed_sec + 1'b1;

385:                     end

386:                 end

387:             end else begin

388:                 one_second_cnt <= one_second_cnt + 1'b1;

389:             end

390:         end

391:     end

392:

393:     always @(posedge clk_pix or negedge rst_n) begin

394:         if (!rst_n) begin

395:             sevenseg_refresh_cnt <= 32'd0;

396:             sevenseg_scan <= 3'd0;

397:         end else if (sevenseg_refresh_cnt >= (SEVENSEG_REFRESH_DIV - 1)) begin

398:             sevenseg_refresh_cnt <= 32'd0;

399:             sevenseg_scan <= sevenseg_scan + 1'b1;

400:         end else begin

401:             sevenseg_refresh_cnt <= sevenseg_refresh_cnt + 1'b1;

402:         end

403:     end

404:

405:     always @(*) begin

406:         sevenseg_digit = 4'hF;

407:         dp = 1'b1;

408:         an = 8'b11111111;

409:

410:         case (sevenseg_scan)

411:             3'd0: begin

412:                 an = 8'b01111111;

413:                 sevenseg_digit = display_lives;

414:             end

415:             3'd1: begin

416:                 an = 8'b10111111;

417:                 sevenseg_digit = display_min;

418:                 dp = 1'b0;

419:             end

420:             3'd2: begin

421:                 an = 8'b11011111;

422:                 sevenseg_digit = display_sec_tens;

423:             end

424:             3'd3: begin

425:                 an = 8'b11101111;

426:                 sevenseg_digit = display_sec_ones;

427:             end

428:             3'd4: begin

429:                 an = 8'b11110111;

430:                 sevenseg_digit = 4'hF;

431:             end

432:             3'd5: begin

433:                 an = 8'b11111011;

434:                 sevenseg_digit = score_hundreds;

435:             end

436:             3'd6: begin

437:                 an = 8'b11111101;

438:                 sevenseg_digit = score_tens;

439:             end

440:             default: begin

441:                 an = 8'b11111110;

442:                 sevenseg_digit = score_ones;

443:             end

444:         endcase

445:

446:         seg = sevenseg_pattern(sevenseg_digit);

447:     end

状态打包给 GPIO/MicroBlaze（vga_snake.srcs/sources_1/new/top.v:484-484）

484:     assign gpio_status = {lives, status_state, score};

10.4 无敌闪烁与重生

玩家受伤但生命未耗尽时会设置 invincible_ticks，并让 invincible_blink 翻转；cell_state 输出阶段用 show_player 控制是否显示玩家，实现闪烁。自撞后还会 pending_player_respawn，扫描安全位置重建蛇身。

无敌计数与闪烁（vga_snake.srcs/sources_1/new/snake_game.v:1018-1023）

1018:                         if (invincible_ticks != 0) begin

1019:                             invincible_ticks <= invincible_ticks - 1'b1;

1020:                             invincible_blink <= ~invincible_blink;

1021:                         end else begin

1022:                             invincible_blink <= 1'b1;

1023:                         end

受伤后进入重生扫描（vga_snake.srcs/sources_1/new/snake_game.v:1060-1073）

1060:                                 end else begin

1061:                                     lives <= lives - 1'b1;

1062:                                     invincible_ticks <= INVINCIBLE_TICKS;

1063:                                     invincible_blink <= 1'b0;

1064:                                     growth_pending <= 0;

1065:                                     pending_player_respawn <= 1'b1;

1066:                                     respawn_scan_idx <= 0;

1067:                                     respawn_scan_x <= random_x(lfsr);

1068:                                     respawn_scan_y <= random_y(lfsr);

1069:                                     respawn_head_dir <= lfsr[1:0];

1070:                                     respawn_path_mode <= lfsr[3:2];

1071:                                     respawn_phase <= RESP_SCAN;

1072:                                     respawn_build_idx <= 0;

1073:                                     respawn_bend_seen <= 1'b0;

11. 总结

本工程的实现思路可以概括为：snake_game 维护离散网格世界，grid_renderer 把网格世界转换为像素颜色，vga_timing 提供显示器需要的扫描时序，top/board_top 完成按键、数码管、GAMEOVER 覆盖和板级连接。随机功能都来自 LFSR；碰撞功能都围绕下一格 nidx、身体位图、敌人数组和食物坐标展开；颜色和显示则通过 cell_state 到 RGB 的两级映射完成。